Dilution Gauging (Gulp Injection) Worksheet

Date:  4 February 1995			Time: 3:30 pm		Operator: NAC/LS/NRT

Site: White Scar, above main fall (40 m mixing reach)		Meter Units:	S cm-1

Background connductivity:  125 S cm-1 					C0:  88 mg l-1

Note: C0 (mg l-1 ) is the background conductivity (S cm-1) at White Scar Cave multiplied by 0.7

Tracer volume (V):  20 l (20 or 40 litres)		Tracer conc (C1):  45,000 mg l-1

Note: 900 g in 20 l is 45,000 mg l-1

Time			Reading			Concentration		C2 - C0

(mins)			(Scm-1)		C2 (mgl-1)		(mgl-1)

__________________________________________________________________________________

0			125			88			0

0.5			125			88			0

1			125			88			0

1.5			125			88			0

2			250			175			87

2.5			417			292			204

3			369			258			170

3.5			274			192			104

4			202			141			53

4.5			163			114			26

5			145			102			14

5.5			135			95			7

6			130			91			3

6.5			128			90			2

7			127			89			1

7.5			126			88			0

8			126			88			0

__________________________________________________________________________________

Note: This example and your field test is an illustration only. An accurate dilution gauging would require: (i) a more precise and accurate conductivity-salt calibration, (ii) a time increment of at least 0.25 mins, rather than the 0.5 mins presented here, and (iii) more precise conversions above.

Notice the long recession in the C2-CO values. This results from the tracer mixing into the large body of slow moving water (a ‘dead-zone’) just below the insurgent waterfall.

The integral of (C2-C0) is:

½ (0.5 mins) (0 + 2 * (87 + 204 + 170 + 104 + 53 + 26 + 14 + 7 + 3 + 2 + 1) + 0)

=	½ (0.5 mins) (0 + 2 (671) + 0)	=	336 mg l-1 min

The discharge is:

20 l x 45,000 mg l-1

_______________________	=	2,679 l min-1		= 	0.045 m3 s-1

336 mg l-1 min